Names: David Fatheree, Preston May

# MAIN/TOP SECTION OF VERILOG CODE

`timescale 1ns / 1ps

module top(

input        i_rx,

input        clk,

output       o_tx,

input        rst

);

// UART signals

reg         tx_start;

wire        rx_valid, tx_done, tx_busy;

reg  [127:0] tx_byte;

wire [255:0] rx_byte;

// UART module

uart_sm #(.BAUD(115200), .CLK_SPEED(100_000_000)) UART_LINK (

.clk(clk),

.reset(rst),

.i_rx(i_rx),

.o_tx(o_tx),

.tx_start(tx_start),

.tx_done(tx_done),

.tx_busy(tx_busy),

.rx_done_really(rx_valid),

.rx_words(rx_byte),

.tx_words(tx_byte)

);

// FSM states

localparam RST      = 2'b00,

WAIT     = 2'b01,

COMPUTE  = 2'b10,

START_TX = 2'b11;

reg [1:0] state;

reg  [127:0] plaintext_reg;

reg  [127:0] key_in;

wire [127:0] ciphertext;

wire [127:0] k0, k1, k2, k3, k4, k5, k6, k7, k8, k9, k10;

key_generator u_keygen (

.key_in(key_in),

.k0(k0), .k1(k1), .k2(k2), .k3(k3), .k4(k4),

.k5(k5), .k6(k6), .k7(k7), .k8(k8), .k9(k9), .k10(k10)

);

wire [127:0] r0_out, r1_out, r2_out, r3_out, r4_out, r5_out, r6_out, r7_out, r8_out, r9_out, r10_out;

assign r0_out = plaintext_reg ^ k0;

aes_round r1 (.clk(clk), .in(r0_out), .key(k1), .out(r1_out));

aes_round r2 (.clk(clk), .in(r1_out), .key(k2), .out(r2_out));

aes_round r3 (.clk(clk), .in(r2_out), .key(k3), .out(r3_out));

aes_round r4 (.clk(clk), .in(r3_out), .key(k4), .out(r4_out));

aes_round r5 (.clk(clk), .in(r4_out), .key(k5), .out(r5_out));

aes_round r6 (.clk(clk), .in(r5_out), .key(k6), .out(r6_out));

aes_round r7 (.clk(clk), .in(r6_out), .key(k7), .out(r7_out));

aes_round r8 (.clk(clk), .in(r7_out), .key(k8), .out(r8_out));

aes_round r9 (.clk(clk), .in(r8_out), .key(k9), .out(r9_out));

aes_final_round r10 (.clk(clk), .in(r9_out), .key(k10), .out(r10_out));

assign ciphertext = r10_out;

// State-Machine logic

always @(posedge clk or posedge rst) begin

if (rst) begin

state <= RST;

tx_start <= 0;

tx_byte <= 0;

plaintext_reg <= 0;

key_in <= 0;

end else begin

tx_start <= 0;

case (state)

RST: begin

if (rx_valid) begin

plaintext_reg <= rx_byte[127:0];

key_in        <= rx_byte[255:128];

state <= COMPUTE;

end

end

COMPUTE: begin

tx_byte <= ciphertext;

state <= START_TX;

end

START_TX: begin

if (!tx_busy) begin

tx_start <= 1'b1;

state <= WAIT;

end

end

WAIT: begin

if (tx_done)

state <= RST;

end

endcase

end

end

endmodule

# PYTHON TEST CODE

import sys, time, serial, random

from AES_Python import AES

SERIAL_PORT = "COM3"

BAUD_RATE = 115200

NUM_TESTS = 20

SBOX = [

0x63,0x7c,0x77,0x7b,0xf2,0x6b,0x6f,0xc5,0x30,0x01,0x67,0x2b,0xfe,0xd7,0xab,0x76,

0xca,0x82,0xc9,0x7d,0xfa,0x59,0x47,0xf0,0xad,0xd4,0xa2,0xaf,0x9c,0xa4,0x72,0xc0,

0xb7,0xfd,0x93,0x26,0x36,0x3f,0xf7,0xcc,0x34,0xa5,0xe5,0xf1,0x71,0xd8,0x31,0x15,

0x04,0xc7,0x23,0xc3,0x18,0x96,0x05,0x9a,0x07,0x12,0x80,0xe2,0xeb,0x27,0xb2,0x75,

0x09,0x83,0x2c,0x1a,0x1b,0x6e,0x5a,0xa0,0x52,0x3b,0xd6,0xb3,0x29,0xe3,0x2f,0x84,

0x53,0xd1,0x00,0xed,0x20,0xfc,0xb1,0x5b,0x6a,0xcb,0xbe,0x39,0x4a,0x4c,0x58,0xcf,

0xd0,0xef,0xaa,0xfb,0x43,0x4d,0x33,0x85,0x45,0xf9,0x02,0x7f,0x50,0x3c,0x9f,0xa8,

0x51,0xa3,0x40,0x8f,0x92,0x9d,0x38,0xf5,0xbc,0xb6,0xda,0x21,0x10,0xff,0xf3,0xd2,

0xcd,0x0c,0x13,0xec,0x5f,0x97,0x44,0x17,0xc4,0xa7,0x7e,0x3d,0x64,0x5d,0x19,0x73,

0x60,0x81,0x4f,0xdc,0x22,0x2a,0x90,0x88,0x46,0xee,0xb8,0x14,0xde,0x5e,0x0b,0xdb,

0xe0,0x32,0x3a,0x0a,0x49,0x06,0x24,0x5c,0xc2,0xd3,0xac,0x62,0x91,0x95,0xe4,0x79,

0xe7,0xc8,0x37,0x6d,0x8d,0xd5,0x4e,0xa9,0x6c,0x56,0xf4,0xea,0x65,0x7a,0xae,0x08,

0xba,0x78,0x25,0x2e,0x1c,0xa6,0xb4,0xc6,0xe8,0xdd,0x74,0x1f,0x4b,0xbd,0x8b,0x8a,

0x70,0x3e,0xb5,0x66,0x48,0x03,0xf6,0x0e,0x61,0x35,0x57,0xb9,0x86,0xc1,0x1d,0x9e,

0xe1,0xf8,0x98,0x11,0x69,0xd9,0x8e,0x94,0x9b,0x1e,0x87,0xe9,0xce,0x55,0x28,0xdf,

0x8c,0xa1,0x89,0x0d,0xbf,0xe6,0x42,0x68,0x41,0x99,0x2d,0x0f,0xb0,0x54,0xbb,0x16

]

RCON = [0x01,0x02,0x04,0x08,0x10,0x20,0x40,0x80,0x1B,0x36]

def sub_bytes(state):

return bytes(SBOX[b] for b in state)

def shift_rows(state_bytes):

s = list(state_bytes)

m = [

[s[0],  s[4],  s[8],  s[12]],  # row 0

[s[1],  s[5],  s[9],  s[13]],  # row 1

[s[2],  s[6],  s[10], s[14]],  # row 2

[s[3],  s[7],  s[11], s[15]],  # row 3

]

out = []

for i, row in enumerate(m):

out.extend(row[i:] + row[:i])

return bytes(out)

def transpose_state(state):

out = bytearray(16)

for r in range(4):

for c in range(4):

out[4 * r + c] = state[4 * c + r]

return bytes(out)

def mul(a, b):

res = 0

for _ in range(8):

if b & 1:

res ^= a

high = a & 0x80

a = (a << 1) & 0xFF

if high:

a ^= 0x1B

b >>= 1

return res

def mix_single_column(col):

a = list(col)

return bytes([

mul(2,a[0]) ^ mul(3,a[1]) ^ a[2] ^ a[3],

a[0] ^ mul(2,a[1]) ^ mul(3,a[2]) ^ a[3],

a[0] ^ a[1] ^ mul(2,a[2]) ^ mul(3,a[3]),

mul(3,a[0]) ^ a[1] ^ a[2] ^ mul(2,a[3])

])

def mix_columns(state):

s = [list(state[i::4]) for i in range(4)]

mixed = [mix_single_column(col) for col in s]

out = bytearray(16)

for c in range(4):

for r in range(4):

out[r + 4*c] = mixed[c][r]

return bytes(out)

def sub_word(w):

return ((SBOX[(w>>24)&0xFF]<<24)|

(SBOX[(w>>16)&0xFF]<<16)|

(SBOX[(w>>8)&0xFF]<<8)|

SBOX[w&0xFF])

def rot_word(w):

return ((w<<8)&0xFFFFFFFF)|(w>>24)

def expand_key(key):

w = [int.from_bytes(key[i:i+4], 'big') for i in range(0, 16, 4)]

for i in range(4, 44):

temp = w[i - 1]

if i % 4 == 0:

temp = sub_word(rot_word(temp)) ^ (RCON[(i // 4) - 1] << 24)

w.append(w[i - 4] ^ temp)

return [b''.join(x.to_bytes(4, 'big') for x in w[i:i+4]) for i in range(0, 44, 4)]

def aes_encrypt(plaintext, key):

round_keys = expand_key(key)

state = bytes(a ^ b for a, b in zip(plaintext, round_keys[0]))

for r in range(1, 10):

state = sub_bytes(state)

state = shift_rows(state)

state = mix_columns(state)

state = bytes(a ^ b for a, b in zip(state, round_keys[r]))

# Final round w/ internal transpose

state = sub_bytes(state)

state = shift_rows(state)

state = transpose_state(state)

state = bytes(a ^ b for a, b in zip(state, round_keys[10]))

return state

def send_and_receive(port, baud, key, plaintext):

data = key + plaintext

with serial.Serial(port,baud,timeout=1.0) as ser:

ser.reset_input_buffer()

ser.reset_output_buffer()

time.sleep(0.02)

ser.write(data)

ser.flush()

return ser.read(16)

def main():

print(f"Testing AES-128 via UART on {SERIAL_PORT} @ {BAUD_RATE}")

total = 0

passed = 0

for i in range(NUM_TESTS):

key=bytes(random.getrandbits(8) for _ in range(16))

plaintext=bytes(random.getrandbits(8) for _ in range(16))

expected = aes_encrypt(plaintext, key)

rx = send_and_receive(SERIAL_PORT, BAUD_RATE, key, plaintext)

print(f"\nTest {i+1:02d}")

print(f"  Key          : {key.hex()}")

print(f"  Plaintext    : {plaintext.hex()}")

print(f"  Expected     : {expected.hex()}")

ok = (len(rx) == 16 and rx == expected)

if len(rx) != 16:

print(f"  Received     : (incomplete, {len(rx)} bytes) {rx.hex()}")

else:

print(f"  Received : {rx.hex()}")

print("  → PASS" if ok else "  → FAIL")

total += 1

passed += ok

print(f"\n{passed}/{NUM_TESTS} tests passed")

if __name__ == "__main__":

main()

# RESULTS SNAPSHOT