Names: David Fatheree, Preston May

`timescale 1ns / 1ps

module top(

input       i_rx,

input       clk,

output      o_tx,

input       rst

);

// UART signals

reg         tx_start;

wire        rx_valid, tx_done, tx_busy;

reg  [127:0] tx_byte;

wire [255:0] rx_byte;

// UART module

uart_sm #(.BAUD(115200), .CLK_SPEED(12_000_000)) UART_LINK (

.clk(clk),

.reset(rst),

.i_rx(i_rx),

.o_tx(o_tx),

.tx_start(tx_start),

.tx_done(tx_done),

.tx_busy(tx_busy),

.rx_done_really(rx_valid),

.rx_words(rx_byte),

.tx_words(tx_byte)

);

// FSM states

localparam RST      = 3'b000,

WAIT     = 3'b001,

COMPUTE  = 3'b010,

START_TX = 3'b011,

WAIT_TX  = 3'b100,

WAIT2    = 3'b101;

reg [2:0] state;

reg [127:0] byte;

reg [127:0] key_in;

wire [127:0] k0, k1;

wire [127:0] dummy[2:10];

key_generator u_keygen (

.key_in(key_in),

.k0(k0), .k1(k1),

.k2(dummy[2]), .k3(dummy[3]), .k4(dummy[4]),

.k5(dummy[5]), .k6(dummy[6]), .k7(dummy[7]),

.k8(dummy[8]), .k9(dummy[9]), .k10(dummy[10])

);

wire [127:0] sub_bytes_out;

wire [127:0] shift_rows_out;

wire [127:0] mix_columns_out;

sub_bytes u_sub_bytes (

.in(byte),

.out(sub_bytes_out)

);

shift_rows u_shift_rows (

.in(sub_bytes_out),

.out(shift_rows_out)

);

mix_columns u_mix_columns (

.in(shift_rows_out),

.out(mix_columns_out)

);

// --- State Machine ---

always @(posedge clk) begin

if (rst) begin

state     <= RST;

byte  <= 128'b0;

key_in    <= 128'b0;

tx_byte   <= 128'b0;

tx_start  <= 1'b0;

end else begin

case (state)

RST: begin

state <= WAIT;

end

WAIT: begin

if (rx_valid) begin

key_in   <= rx_byte[255:128];

byte     <= rx_byte[127:0];

state    <= WAIT2;

end

end

WAIT2: begin

state <= COMPUTE;

end

COMPUTE: begin

tx_byte <= mix_columns_out ^ k1; // AddRoundKey

state   <= START_TX;

end

START_TX: begin

tx_start <= 1'b1;

state <= WAIT_TX;

end

WAIT_TX: begin

tx_start <= 1'b0;

if (tx_done)

state <= RST;

end

endcase

end

end

endmodule

`timescale 1ns / 1ps

module key_generator(

input  [127:0] key_in,

output [127:0] k0,

output [127:0] k1,

output [127:0] k2,

output [127:0] k3,

output [127:0] k4,

output [127:0] k5,

output [127:0] k6,

output [127:0] k7,

output [127:0] k8,

output [127:0] k9,

output [127:0] k10

);

// Round constants

localparam [7:0] RC1  = 8'h01;

localparam [7:0] RC2  = 8'h02;

localparam [7:0] RC3  = 8'h04;

localparam [7:0] RC4  = 8'h08;

localparam [7:0] RC5  = 8'h10;

localparam [7:0] RC6  = 8'h20;

localparam [7:0] RC7  = 8'h40;

localparam [7:0] RC8  = 8'h80;

localparam [7:0] RC9  = 8'h1B;

localparam [7:0] RC10 = 8'h36;

// Round key 0 = original key

assign k0 = key_in;

// Sequential key expansion chain

key_schedule r1  (.in(k0),  .RC(RC1),  .out(k1));

key_schedule r2  (.in(k1),  .RC(RC2),  .out(k2));

key_schedule r3  (.in(k2),  .RC(RC3),  .out(k3));

key_schedule r4  (.in(k3),  .RC(RC4),  .out(k4));

key_schedule r5  (.in(k4),  .RC(RC5),  .out(k5));

key_schedule r6  (.in(k5),  .RC(RC6),  .out(k6));

key_schedule r7  (.in(k6),  .RC(RC7),  .out(k7));

key_schedule r8  (.in(k7),  .RC(RC8),  .out(k8));

key_schedule r9  (.in(k8),  .RC(RC9),  .out(k9));

key_schedule r10 (.in(k9),  .RC(RC10), .out(k10));

endmodule

module key_schedule(

input [127:0] in,

input [7:0] RC,

output [127:0] out

);

wire [31:0] w0 = in[127:96];

wire [31:0] w1 = in[95:64];

wire [31:0] w2 = in[63:32];

wire [31:0] w3 = in[31:0];

wire [31:0] w3_g;

g g_module(.in(w3), .RC(RC), .out(w3_g));

wire [31:0] w0_out = w0 ^ w3_g;

wire [31:0] w1_out = w1 ^ w0_out;

wire [31:0] w2_out = w2 ^ w1_out;

wire [31:0] w3_out = w3 ^ w2_out;

assign out = {w0_out, w1_out, w2_out, w3_out};

endmodule

module g(

input [31:0] in,

input [7:0] RC,

output [31:0] out

);

wire [7:0] v [0:3];

assign { v[0], v[1], v[2], v[3] } = in;

wire [7:0] s0, s1, s2, s3;

s_box sb0 (v[1], s0);

s_box sb1 (v[2], s1);

s_box sb2 (v[3], s2);

s_box sb3 (v[0], s3);

assign out = {s0 ^ RC, s1, s2, s3};

endmodule

`timescale 1ns/1ps

module tb_key_generator;

// DUT inputs/outputs

reg  [127:0] key_in;

wire [127:0] k0, k1, k2, k3, k4, k5, k6, k7, k8, k9, k10;

// Instantiate Device Under Test

key_generator dut (

.key_in(key_in),

.k0(k0), .k1(k1), .k2(k2), .k3(k3), .k4(k4),

.k5(k5), .k6(k6), .k7(k7), .k8(k8), .k9(k9), .k10(k10)

);

// Expected round keys for AES-128 with key:

// 2b7e151628aed2a6abf7158809cf4f3c

// Source: FIPS-197, App. A.1 (words grouped into 128-bit round keys)

localparam [127:0] EXP_K0  = 128'h2b7e1516_28aed2a6_abf71588_09cf4f3c;

localparam [127:0] EXP_K1  = 128'ha0fafe17_88542cb1_23a33939_2a6c7605;

localparam [127:0] EXP_K2  = 128'hf2c295f2_7a96b943_5935807a_7359f67f;

localparam [127:0] EXP_K3  = 128'h3d80477d_4716fe3e_1e237e44_6d7a883b;

localparam [127:0] EXP_K4  = 128'hef44a541_a8525b7f_b671253b_db0bad00;

localparam [127:0] EXP_K5  = 128'hd4d1c6f8_7c839d87_caf2b8bc_11f915bc;

localparam [127:0] EXP_K6  = 128'h6d88a37a_110b3efd_dbf98641_ca0093fd;

localparam [127:0] EXP_K7  = 128'h4e54f70e_5f5fc9f3_84a64fb2_4ea6dc4f;

localparam [127:0] EXP_K8  = 128'head27321_b58dbad2_312bf560_7f8d292f;

localparam [127:0] EXP_K9  = 128'hac7766f3_19fadc21_28d12941_575c006e;

localparam [127:0] EXP_K10 = 128'hd014f9a8_c9ee2589_e13f0cc8_b6630ca6;

integer errors;

// Simple compare task to reduce boilerplate

task check_eq;

input [127:0] got;

input [127:0] exp;

input [8*8:1] name; // small string literal like "k0"

begin

if (got !== exp) begin

errors = errors + 1;

$display("[%0t] ERROR: %s mismatch", $time, name);

$display("        Got: %032h", got);

$display("        Exp: %032h", exp);

end else begin

$display("[%0t] OK   : %s = %032h", $time, name, got);

end

end

endtask

initial begin

errors = 0;

// Drive the canonical AES-128 key

key_in = 128'h2b7e1516_28aed2a6_abf71588_09cf4f3c;

// Wait a delta cycle for combinational propagation

#1;

// Check all round keys

check_eq(k0 , EXP_K0 , "k0" );

check_eq(k1 , EXP_K1 , "k1" );

check_eq(k2 , EXP_K2 , "k2" );

check_eq(k3 , EXP_K3 , "k3" );

check_eq(k4 , EXP_K4 , "k4" );

check_eq(k5 , EXP_K5 , "k5" );

check_eq(k6 , EXP_K6 , "k6" );

check_eq(k7 , EXP_K7 , "k7" );

check_eq(k8 , EXP_K8 , "k8" );

check_eq(k9 , EXP_K9 , "k9" );

check_eq(k10, EXP_K10, "k10");

// Final report

if (errors == 0) begin

$display("------------------------------------------------------------");

$display("ALL TESTS PASSED: key_generator matches FIPS-197 (AES-128).");

$display("------------------------------------------------------------");

end else begin

$display("------------------------------------------------------------");

$display("TESTS FAILED: %0d mismatches detected.", errors);

$display("------------------------------------------------------------");

end

// End simulation

#5;

$finish;

end

endmodule

`timescale 1ns/1ps

module tp_module_top_replica (

input              clk,

input              rst,

input              start,

input      [127:0] key_in,

input      [127:0] byte_in,

output reg [127:0] tx_word,

output reg         tx_word_valid

);

// Key expansion

wire [127:0] k0, k1;

wire [127:0] dummy [2:10];

key_generator u_keygen (

.key_in(key_in),

.k0(k0), .k1(k1),

.k2(dummy[2]), .k3(dummy[3]), .k4(dummy[4]),

.k5(dummy[5]), .k6(dummy[6]), .k7(dummy[7]),

.k8(dummy[8]), .k9(dummy[9]), .k10(dummy[10])

);

// AES first-round data path (as in top.v)

wire [127:0] sub_bytes_out;

wire [127:0] shift_rows_out;

wire [127:0] mix_columns_out;

sub_bytes  u_sub_bytes  (.in(byte_in),        .out(sub_bytes_out));

shift_rows u_shift_rows (.in(sub_bytes_out),  .out(shift_rows_out));

mix_columns u_mix_cols  (.in(shift_rows_out), .out(mix_columns_out));

// Latch result on 'start' to mirror COMPUTE -> START_TX handoff point

always @(posedge clk) begin

if (rst) begin

tx_word       <= 128'b0;

tx_word_valid <= 1'b0;

end else begin

tx_word_valid <= 1'b0;

if (start) begin

tx_word       <= mix_columns_out ^ k1;

tx_word_valid <= 1'b1;

end

end

end

endmodule

// -----------------------------------------------------------------------------

// Testbench: tb_top_sans_uart

// Drives key and plaintext directly.

// Prints the 128-bit tx_word produced by the replicated pipeline.

// -----------------------------------------------------------------------------

module tb_top_all_modules;

// Clock/reset

reg clk;

reg rst;

// Stimulus

reg         start;

reg [127:0] key_in;

reg [127:0] byte_in;

// Outputs from DUT

wire [127:0] tx_word;

wire         tx_word_valid;

tp_module_top_replica dut (

.clk(clk),

.rst(rst),

.start(start),

.key_in(key_in),

.byte_in(byte_in),

.tx_word(tx_word),

.tx_word_valid(tx_word_valid)

);

localparam integer CLK_HZ     = 12_000_000;

localparam integer CLK_PERIOD = 1_000_000_000 / CLK_HZ; // ns

initial begin

clk = 1'b0;

forever #(CLK_PERIOD/2) clk = ~clk;

end

initial begin

#10_000_000;

$display("[%0t] TIMEOUT: ending simulation via watchdog.", $time);

$finish;

end

// Pretty-print helper

task print128;

input [127:0] v;

integer i;

reg [7:0] b [0:15];

begin

{b[0], b[1], b[2], b[3], b[4], b[5], b[6], b[7],

b[8], b[9], b[10], b[11], b[12], b[13], b[14], b[15]} = v;

$write("0x");

for (i = 0; i < 16; i = i + 1) $write("%02x", b[i]);

end

endtask

initial begin

// Reset

start   = 1'b0;

key_in  = 128'b0;

byte_in = 128'b0;

rst = 1'b1;

repeat (8) @(posedge clk);

rst = 1'b0;

repeat (4) @(posedge clk);

// ---------------- Test Case 1 ----------------

key_in  = 128'h2b7e1516_28aed2a6_abf71588_09cf4f3c;

byte_in = 128'h3243f6a8_885a308d_313198a2_e0370734;

// Pulse 'start' for 1 cycle

@(posedge clk); start <= 1'b1;

@(posedge clk); start <= 1'b0;

wait (tx_word_valid === 1'b1);

#1;

$write("[%0t] TX_WORD (Test 1) = ", $time);

print128(tx_word);

$write("\n");

// ---------------- Test Case 2 ----------------

key_in  = 128'h00000000_00000000_00000000_00000000;

byte_in = 128'h00000000_00000000_00000000_00000000;

@(posedge clk); start <= 1'b1;

@(posedge clk); start <= 1'b0;

wait (tx_word_valid === 1'b1);

#1;

$write("[%0t] TX_WORD (Test 2) = ", $time);

print128(tx_word);

$write("\n");

// Grace delay then finish

#(10*CLK_PERIOD);

$display("[%0t] TB done. Calling $finish.", $time);

$finish;

end

endmodule

import sys, time, serial, random

SERIAL_PORT = "COM3"

BAUD_RATE = 115200

NUM_TESTS = 20

SBOX = [

0x63,0x7c,0x77,0x7b,0xf2,0x6b,0x6f,0xc5,0x30,0x01,0x67,0x2b,0xfe,0xd7,0xab,0x76,

0xca,0x82,0xc9,0x7d,0xfa,0x59,0x47,0xf0,0xad,0xd4,0xa2,0xaf,0x9c,0xa4,0x72,0xc0,

0xb7,0xfd,0x93,0x26,0x36,0x3f,0xf7,0xcc,0x34,0xa5,0xe5,0xf1,0x71,0xd8,0x31,0x15,

0x04,0xc7,0x23,0xc3,0x18,0x96,0x05,0x9a,0x07,0x12,0x80,0xe2,0xeb,0x27,0xb2,0x75,

0x09,0x83,0x2c,0x1a,0x1b,0x6e,0x5a,0xa0,0x52,0x3b,0xd6,0xb3,0x29,0xe3,0x2f,0x84,

0x53,0xd1,0x00,0xed,0x20,0xfc,0xb1,0x5b,0x6a,0xcb,0xbe,0x39,0x4a,0x4c,0x58,0xcf,

0xd0,0xef,0xaa,0xfb,0x43,0x4d,0x33,0x85,0x45,0xf9,0x02,0x7f,0x50,0x3c,0x9f,0xa8,

0x51,0xa3,0x40,0x8f,0x92,0x9d,0x38,0xf5,0xbc,0xb6,0xda,0x21,0x10,0xff,0xf3,0xd2,

0xcd,0x0c,0x13,0xec,0x5f,0x97,0x44,0x17,0xc4,0xa7,0x7e,0x3d,0x64,0x5d,0x19,0x73,

0x60,0x81,0x4f,0xdc,0x22,0x2a,0x90,0x88,0x46,0xee,0xb8,0x14,0xde,0x5e,0x0b,0xdb,

0xe0,0x32,0x3a,0x0a,0x49,0x06,0x24,0x5c,0xc2,0xd3,0xac,0x62,0x91,0x95,0xe4,0x79,

0xe7,0xc8,0x37,0x6d,0x8d,0xd5,0x4e,0xa9,0x6c,0x56,0xf4,0xea,0x65,0x7a,0xae,0x08,

0xba,0x78,0x25,0x2e,0x1c,0xa6,0xb4,0xc6,0xe8,0xdd,0x74,0x1f,0x4b,0xbd,0x8b,0x8a,

0x70,0x3e,0xb5,0x66,0x48,0x03,0xf6,0x0e,0x61,0x35,0x57,0xb9,0x86,0xc1,0x1d,0x9e,

0xe1,0xf8,0x98,0x11,0x69,0xd9,0x8e,0x94,0x9b,0x1e,0x87,0xe9,0xce,0x55,0x28,0xdf,

0x8c,0xa1,0x89,0x0d,0xbf,0xe6,0x42,0x68,0x41,0x99,0x2d,0x0f,0xb0,0x54,0xbb,0x16

]

def sub_bytes(state_bytes):

return bytes(SBOX[b] for b in state_bytes)

def shift_rows(state_bytes):

s = list(state_bytes)

m = [

[s[0],  s[4],  s[8],  s[12]],  # row 0

[s[1],  s[5],  s[9],  s[13]],  # row 1

[s[2],  s[6],  s[10], s[14]],  # row 2

[s[3],  s[7],  s[11], s[15]],  # row 3

]

out = []

for i, row in enumerate(m):

out.extend(row[i:] + row[:i])

return bytes(out)

def mul(a, b):

res = 0

for _ in range(8):

if b & 1:

res ^= a

high = a & 0x80

a = (a << 1) & 0xFF

if high:

a ^= 0x1B

b >>= 1

return res

def mix_single_column(col):

a = list(col)

return bytes([

mul(2,a[0]) ^ mul(3,a[1]) ^ a[2] ^ a[3],

a[0] ^ mul(2,a[1]) ^ mul(3,a[2]) ^ a[3],

a[0] ^ a[1] ^ mul(2,a[2]) ^ mul(3,a[3]),

mul(3,a[0]) ^ a[1] ^ a[2] ^ mul(2,a[3])

])

def mix_columns(state):

s = [list(state[i::4]) for i in range(4)]

mixed = [mix_single_column(col) for col in s]

out = bytearray(16)

for c in range(4):

for r in range(4):

out[r + 4*c] = mixed[c][r]

return bytes(out)

def sub_word(w):

return ((SBOX[(w>>24)&0xFF]<<24)|

(SBOX[(w>>16)&0xFF]<<16)|

(SBOX[(w>>8)&0xFF]<<8)|

SBOX[w&0xFF])

def rot_word(w):

return ((w<<8)&0xFFFFFFFF)|(w>>24)

def next_round_key(key_bytes, RC=0x01):

w=[int.from_bytes(key_bytes[i:i+4],'big') for i in range(0,16,4)]

temp=sub_word(rot_word(w[3]))^(RC<<24)

w4=w[0]^temp

w5=w[1]^w4

w6=w[2]^w5

w7=w[3]^w6

return b''.join(x.to_bytes(4,'big') for x in [w4,w5,w6,w7])

def one_round_encrypt(state,key):

k1=next_round_key(key)

s=sub_bytes(state)

s=shift_rows(s)

s=mix_columns(s)

return bytes(a^b for a,b in zip(s,k1))

def send_and_receive(port,baud,data):

with serial.Serial(port,baud,timeout=1.0) as ser:

ser.reset_input_buffer()

ser.reset_output_buffer()

time.sleep(0.02)

ser.write(data)

ser.flush()

return ser.read(16)

def main():

print(f"Testing one AES round via UART on {SERIAL_PORT} @ {BAUD_RATE}")

total = 0

passed = 0

for i in range(NUM_TESTS):

key=bytes(random.getrandbits(8) for _ in range(16))

plaintext=bytes(random.getrandbits(8) for _ in range(16))

expected=one_round_encrypt(plaintext,key)

rx=send_and_receive(SERIAL_PORT,BAUD_RATE,key+plaintext)

print(f"\nTest {i+1:02d}")

print(f"  Key          : {key.hex()}")

print(f"  Plaintext    : {plaintext.hex()}")

print(f"  Expected     : {expected.hex()}")

ok = (len(rx) == 16 and rx == expected)

if len(rx) != 16:

print(f"  Received     : (incomplete, {len(rx)} bytes) {rx.hex()}")

else:

print(f"  Received : {rx.hex()}")

print("  → PASS" if ok else "  → FAIL")

total += 1

passed += ok

print(f"\n{passed}/{NUM_TESTS} tests passed")

if __name__=="__main__":

main()

[1000] OK   :       k0 = 2b7e151628aed2a6abf7158809cf4f3c

[1000] OK   :       k1 = a0fafe1788542cb123a339392a6c7605

[1000] OK   :       k2 = f2c295f27a96b9435935807a7359f67f

[1000] OK   :       k3 = 3d80477d4716fe3e1e237e446d7a883b

[1000] OK   :       k4 = ef44a541a8525b7fb671253bdb0bad00

[1000] OK   :       k5 = d4d1c6f87c839d87caf2b8bc11f915bc

[1000] OK   :       k6 = 6d88a37a110b3efddbf98641ca0093fd

[1000] OK   :       k7 = 4e54f70e5f5fc9f384a64fb24ea6dc4f

[1000] OK   :       k8 = ead27321b58dbad2312bf5607f8d292f

[1000] OK   :       k9 = ac7766f319fadc2128d12941575c006e

[1000] OK   :      k10 = d014f9a8c9ee2589e13f0cc8b6630ca6

------------------------------------------------------------

ALL TESTS PASSED: key_generator matches FIPS-197 (AES-128).

[1108000] TX_WORD (Test 1) = 0x616cc7ba4e93e37b1cd00709e38fcb57

[1272000] TX_WORD (Test 2) = 0x01000000010000000100000001000000

[2102000] TB done. Calling $finish.