NAMES: David Fatheree, Preston May

`timescale 1ns / 1ps

module top(

input       i_rx,

input       clk,

output      o_tx,

input       rst

);

// UART signals

reg         tx_start;

wire        rx_valid, tx_done, tx_busy;

reg [127:0] tx_byte;

wire [127:0] rx_byte;

// UART module

uart_sm #(.BAUD(115200), .CLK_SPEED(12_000_000)) UART_LINK (

.clk(clk),

.reset(rst),

.i_rx(i_rx),

.o_tx(o_tx),

.tx_start(tx_start),

.tx_done(tx_done),

.tx_busy(tx_busy),

.rx_done_really(rx_valid),

.rx_words(rx_byte),

.tx_words(tx_byte)

);

// FSM states

localparam RST      = 3'b000,

WAIT     = 3'b001,

COMPUTE  = 3'b010,

START_TX = 3'b011,

WAIT_TX  = 3'b100,

WAIT2    = 3'b101;

reg [2:0] state;

reg [127:0] byte;

wire [127:0] sub_bytes_out;

sub_bytes u_sub_bytes (

.in(byte),

.out(sub_bytes_out)

);

wire [127:0] shift_rows_out;

shift_rows u_shift_rows (

.in(sub_bytes_out),

.out(shift_rows_out)

);

wire [127:0] mc_out;

mix_columns u_mix_columns (

.in(shift_rows_out),

.out(mc_out)

);

// --- State Machine ---

always @(posedge clk) begin

if (rst) begin

state     <= RST;

byte      <= 128'b0;

tx_byte   <= 128'b0;

tx_start  <= 1'b0;

end else begin

case (state)

RST: begin

state <= WAIT;

end

WAIT: begin

if (rx_valid) begin

byte  <= rx_byte;

state <= WAIT2;

end

end

WAIT2: begin

state <= COMPUTE;

end

COMPUTE: begin

tx_byte <= mc_out;  // Send the full output

state   <= START_TX;

end

START_TX: begin

tx_start <= 1'b1;

state <= WAIT_TX;

end

WAIT_TX: begin

tx_start <= 1'b0;

if (tx_done)

state <= RST;

end

endcase

end

end

endmodule

`timescale 1ns/1ps

module tb_shift_rows;

reg  [127:0] in;

wire [127:0] out;

// DUT

shift_rows dut (

.in(in),

.out(out)

);

function [127:0] ref_shift_rows;

input [127:0] x;

reg [7:0] s [0:15];

begin

{ s[0],  s[1],  s[2],  s[3],

s[4],  s[5],  s[6],  s[7],

s[8],  s[9],  s[10], s[11],

s[12], s[13], s[14], s[15] } = x;

ref_shift_rows = {

s[0],  s[4],  s[8],  s[12],

s[5],  s[9],  s[13], s[1],

s[10], s[14], s[2],  s[6],

s[15], s[3],  s[7],  s[11]

};

end

endfunction

integer i;

reg [127:0] exp;

initial begin

// Test 1: Identity-friendly ramp bytes 0x00..0x0F placed as {b0..b15}

in = 128'h00_01_02_03_04_05_06_07_08_09_0A_0B_0C_0D_0E_0F;

#1;

exp = ref_shift_rows(in);

if (out !== exp) begin

$display("FAIL T1: out=%032h exp=%032h", out, exp);

$fatal;

end else begin

$display("PASS T1: out=%032h", out);

end

// Test 2: Random-looking pattern

in = 128'hD4_E0_B8_1E_BF_B4_41_27_5D_52_11_98_30_AE_F1_E5;

#1;

exp = ref_shift_rows(in);

if (out !== exp) begin

$display("FAIL T2: out=%032h exp=%032h", out, exp);

$fatal;

end else begin

$display("PASS T2: out=%032h", out);

end

// Test 3: All same byte

in = {16{8'hAA}};

#1;

exp = ref_shift_rows(in);

if (out !== exp) begin

$display("FAIL T3: out=%032h exp=%032h", out, exp);

$fatal;

end else begin

$display("PASS T3: out=%032h", out);

end

$display("All shift_rows tests passed.");

$finish;

end

endmodule

`timescale 1ns/1ps

module tb_mix_column;

reg  [31:0] in_col;

wire [31:0] out_col;

// DUT

mix_column dut (

.in(in_col),

.out(out_col)

);

// Helper: pretty print a 32-bit column as four bytes

task show_vec;

input [31:0] v;

begin

$display("%02h %02h %02h %02h", v[31:24], v[23:16], v[15:8], v[7:0]);

end

endtask

// Known-good GF(2^8) MixColumns example from FIPS-197:

// Input column [db 13 53 45] -> Output [8e 4d a1 bc]

// Note: The module declares in as {a0,a1,a2,a3} where a0 is the MSB byte.

// We'll supply in_col = 32'hdb_13_53_45 and expect 32'h8e_4d_a1_bc.

initial begin

// Test 1: FIPS-197 example 1

in_col = 32'hdb_13_53_45;

#1;

if (out_col !== 32'h8e_4d_a1_bc) begin

$display("FAIL T1: got=");

show_vec(out_col);

$display(" exp=8e 4d a1 bc");

$fatal;

end else begin

$display("PASS T1: FIPS column ok");

end

// Test 2: All ones -> remains all ones

// Matrix*[01 01 01 01]^T = [01 01 01 01]^T

in_col = {4{8'h01}}; // 32'h01_01_01_01

#1;

if (out_col !== {4{8'h01}}) begin

$display("FAIL T2: got=");

show_vec(out_col);

$display(" exp=01 01 01 01");

$fatal;

end else begin

$display("PASS T2: all-ones column ok");

end

// Test 3: All zeros -> all zeros

in_col = 32'h00_00_00_00;

#1;

if (out_col !== 32'h00_00_00_00) begin

$display("FAIL T3");

$fatal;

end else begin

$display("PASS T3: all-zeros column ok");

end

$display("All mix_column tests passed.");

$finish;

end

endmodule

`timescale 1ns/1ps

// -----------------------------------------------------------------------------

// DUT wrapper: one AES "round core" without AddRoundKey

// state_out = MixColumns( ShiftRows( SubBytes(state_in) ) )

// -----------------------------------------------------------------------------

module round_chain (

input  wire [127:0] state_in,

output wire [127:0] state_out

);

// SubBytes: 16 parallel S-boxes, byte-wise

wire [127:0] subbytes_out;

genvar i;

generate

for (i = 0; i < 16; i = i + 1) begin : G_SBOX

wire [7:0] in_b  = state_in[127 - 8*i -: 8];

wire [7:0] out_b;

s_box u_sbox (.in_byte(in_b), .out_byte(out_b));

assign subbytes_out[127 - 8*i -: 8] = out_b;

end

endgenerate

// ShiftRows

wire [127:0] sr_out;

shift_rows U_SR (.in(subbytes_out), .out(sr_out));

// MixColumns (128-bit variant that applies mix_column per 32-bit column)

wire [127:0] mc_out;

mix_columns U_MC (.in(sr_out), .out(mc_out));

assign state_out = mc_out;

endmodule

// -----------------------------------------------------------------------------

// Testbench

// -----------------------------------------------------------------------------

module tb_all_together;

// No clock required: the chain is purely combinational

reg  [127:0] in_state;

wire [127:0] out_state;

// DUT: only the three transforms

round_chain DUT (

.state_in(in_state),

.state_out(out_state)

);

// Pretty-print a 128-bit state as 16 bytes

task show_state;

input [127:0] s;

begin

$display("%02h %02h %02h %02h  %02h %02h %02h %02h  %02h %02h %02h %02h  %02h %02h %02h %02h",

s[127:120], s[119:112], s[111:104], s[103:96],

s[95:88],   s[87:80],   s[79:72],   s[71:64],

s[63:56],   s[55:48],   s[47:40],   s[39:32],

s[31:24],   s[23:16],   s[15:8],    s[7:0]);

end

endtask

// Simple "not-X" check

task assert_no_x;

input [127:0] v;

input [127:0] exp_mask;

begin

if ((^v) === 1'bx) begin

$display("FAIL: X detected in vector!");

$fatal;

end

end

endtask

initial begin

$display("==== tb_top (SubBytes -> ShiftRows -> MixColumns) ====");

// Test 1: byte ramp

in_state = 128'h00_01_02_03_04_05_06_07_08_09_0A_0B_0C_0D_0E_0F;

#1;

$display("T1 in :"); show_state(in_state);

$display("T1 out:"); show_state(out_state);

assert_no_x(out_state, 128'hFFFF_FFFF_FFFF_FFFF_FFFF_FFFF_FFFF_FFFF);

// Test 2: FIPS-like pattern (used in many AES examples)

in_state = 128'hD4_E0_B8_1E_BF_B4_41_27_5D_52_11_98_30_AE_F1_E5;

#1;

$display("T2 in :"); show_state(in_state);

$display("T2 out:"); show_state(out_state);

assert_no_x(out_state, 128'hFFFF_FFFF_FFFF_FFFF_FFFF_FFFF_FFFF_FFFF);

// Test 3: all equal bytes

in_state = {16{8'hAA}};

#1;

$display("T3 in :"); show_state(in_state);

$display("T3 out:"); show_state(out_state);

assert_no_x(out_state, 128'hFFFF_FFFF_FFFF_FFFF_FFFF_FFFF_FFFF_FFFF);

$display("tb_top completed (visual check).");

$finish;

end

endmodule

import sys

import time

import serial

import random

SERIAL_PORT = "COM3"   #check device manager and change to that com port

BAUD_RATE = 115200

NUM_TESTS   = 20

SBOX = [

0x63,0x7c,0x77,0x7b,0xf2,0x6b,0x6f,0xc5,0x30,0x01,0x67,0x2b,0xfe,0xd7,0xab,0x76,

0xca,0x82,0xc9,0x7d,0xfa,0x59,0x47,0xf0,0xad,0xd4,0xa2,0xaf,0x9c,0xa4,0x72,0xc0,

0xb7,0xfd,0x93,0x26,0x36,0x3f,0xf7,0xcc,0x34,0xa5,0xe5,0xf1,0x71,0xd8,0x31,0x15,

0x04,0xc7,0x23,0xc3,0x18,0x96,0x05,0x9a,0x07,0x12,0x80,0xe2,0xeb,0x27,0xb2,0x75,

0x09,0x83,0x2c,0x1a,0x1b,0x6e,0x5a,0xa0,0x52,0x3b,0xd6,0xb3,0x29,0xe3,0x2f,0x84,

0x53,0xd1,0x00,0xed,0x20,0xfc,0xb1,0x5b,0x6a,0xcb,0xbe,0x39,0x4a,0x4c,0x58,0xcf,

0xd0,0xef,0xaa,0xfb,0x43,0x4d,0x33,0x85,0x45,0xf9,0x02,0x7f,0x50,0x3c,0x9f,0xa8,

0x51,0xa3,0x40,0x8f,0x92,0x9d,0x38,0xf5,0xbc,0xb6,0xda,0x21,0x10,0xff,0xf3,0xd2,

0xcd,0x0c,0x13,0xec,0x5f,0x97,0x44,0x17,0xc4,0xa7,0x7e,0x3d,0x64,0x5d,0x19,0x73,

0x60,0x81,0x4f,0xdc,0x22,0x2a,0x90,0x88,0x46,0xee,0xb8,0x14,0xde,0x5e,0x0b,0xdb,

0xe0,0x32,0x3a,0x0a,0x49,0x06,0x24,0x5c,0xc2,0xd3,0xac,0x62,0x91,0x95,0xe4,0x79,

0xe7,0xc8,0x37,0x6d,0x8d,0xd5,0x4e,0xa9,0x6c,0x56,0xf4,0xea,0x65,0x7a,0xae,0x08,

0xba,0x78,0x25,0x2e,0x1c,0xa6,0xb4,0xc6,0xe8,0xdd,0x74,0x1f,0x4b,0xbd,0x8b,0x8a,

0x70,0x3e,0xb5,0x66,0x48,0x03,0xf6,0x0e,0x61,0x35,0x57,0xb9,0x86,0xc1,0x1d,0x9e,

0xe1,0xf8,0x98,0x11,0x69,0xd9,0x8e,0x94,0x9b,0x1e,0x87,0xe9,0xce,0x55,0x28,0xdf,

0x8c,0xa1,0x89,0x0d,0xbf,0xe6,0x42,0x68,0x41,0x99,0x2d,0x0f,0xb0,0x54,0xbb,0x16

]

def sub_bytes(state_bytes):

return bytes(SBOX[b] for b in state_bytes)

def shift_rows(state_bytes):

s = list(state_bytes)

m = [

[s[0],  s[4],  s[8],  s[12]],  # row 0

[s[1],  s[5],  s[9],  s[13]],  # row 1

[s[2],  s[6],  s[10], s[14]],  # row 2

[s[3],  s[7],  s[11], s[15]],  # row 3

]

out = []

for i, row in enumerate(m):

out.extend(row[i:] + row[:i])

return bytes(out)

def mul(a, b):

res = 0

for _ in range(8):

if b & 1:

res ^= a

high = a & 0x80

a = (a << 1) & 0xFF

if high:

a ^= 0x1B

b >>= 1

return res

def mix_single_column(col):

a = list(col)

return bytes([

mul(2,a[0]) ^ mul(3,a[1]) ^ a[2] ^ a[3],

a[0] ^ mul(2,a[1]) ^ mul(3,a[2]) ^ a[3],

a[0] ^ a[1] ^ mul(2,a[2]) ^ mul(3,a[3]),

mul(3,a[0]) ^ a[1] ^ a[2] ^ mul(2,a[3])

])

def mix_columns(state):

s = [list(state[i::4]) for i in range(4)]

mixed = [mix_single_column(col) for col in s]

out = bytearray(16)

for c in range(4):

for r in range(4):

out[r + 4*c] = mixed[c][r]

return bytes(out)

def send_and_receive(port, baud, data, timeout=1.0):

with serial.Serial(port, baud, timeout=timeout) as ser:

ser.reset_input_buffer()

ser.reset_output_buffer()

time.sleep(0.02)

ser.write(data)

ser.flush()

time.sleep(0.02)

rx = ser.read(16)

return rx

def aes_sub_shift_mix(state):

return mix_columns(shift_rows(sub_bytes(state)))

# --- Main test ---

def main():

print(f"Testing SubBytes -> ShiftRows -> MixColumns chain on {SERIAL_PORT} @ {BAUD_RATE}")

total = 0

passed = 0

for i in range(NUM_TESTS):

#random 16-byte input

input_state = bytes(random.getrandbits(8) for _ in range(16))

#expected reference output

expected = aes_sub_shift_mix(input_state)

rx = send_and_receive(SERIAL_PORT, BAUD_RATE, input_state)

ok = (len(rx) == 16 and rx == expected)

print(f"\nTest {i+1:02d}")

print(f"  Input    : {input_state.hex()}")

print(f"  Expected : {expected.hex()}")

if len(rx) != 16:

print(f"  Received : (incomplete, {len(rx)} bytes) {rx.hex()}")

else:

print(f"  Received : {rx.hex()}")

print("  → PASS" if ok else "  → FAIL")

total += 1

passed += ok

print(f"\nSummary: {passed}/{total} passed ({100*passed/total:.1f}%)")

if __name__ == "__main__":

main()